# 📄 Secure Document Destruction Solution - for Professional Operators

## Integrated Shredding Lines - 2,000 Tons / Year

### 1. Project Overview

This solution is designed for professional document destruction operators providing outsourced services to banks and financial institutions.

The system focuses on long-term operational stability, security compliance, and cost efficiency, which are critical for service providers managing continuous workloads from multiple banking clients.

- Total annual volume: approx. 2,000 tons

  - Normal documents: 1,500 tons / year

  - High-security documents (EU P6–P7): 500 tons / year

- Operational assumption:

  - 200 working days per year

  - 6 operating hours per day

- Site condition:

  - Centralized operation at one facility

  - Available area ≥ 500 m², height approx. 6 m

- Design objective:

  - Stable, scalable operation for outsourced destruction services

  - Clear separation of security levels for different banking requirements

### 2. Overall System Concept

The proposed system is designed specifically for operators, not for one-time or internal bank use.

- Normal and high-security documents are processed on separate, dedicated lines

- Parallel line architecture ensures:

  - Continuous operation

  - Easy maintenance without full shutdown

  - Lower operational risk for service providers

- The system is suitable for multi-client, long-term service contracts

## 3. Line A – Normal Document Shredding (EU P2–P3)

Annual volume: approx. 1,500 tons

- Required throughput: 1.2–1.3 tons/hour

- Configuration: 2 parallel shredding lines

Each line includes:

- Primary shredder

  - Motor power: 15 kW

- Secondary shredder

  - Motor power: 10 kW

  - Security level: EU P2–P3

- Single-line capacity: 600–700 kg/hour

Discharge & baling:

- Shredded paper is sent directly to the customer’s existing baler

- Large paper bales for easy handling and resale

- No additional baler investment required

## 4. Line B – High-Security Document Shredding (EU P6–P7)

Annual volume: approx. 500 tons

- Required throughput: approx. 400–420 kg/hour

- Configuration: 2 parallel high-security shredding lines

Each line includes:

- Conveyor feeding system

- Primary shredder

  - Motor power: 11 kW

- High-security fine shredder

  - Motor power: 22 kW

  - Output size: 1–2 mm particles

  - Security level: EU P6–P7

- Single-line capacity: 150–200 kg/hour

Final treatment:

- Shredded paper powder is processed by a hydraulic briquetting system

- Compressed into small cylindrical briquettes

## 5. Key Advantages for Operators

- ✔ Designed for outsourced document destruction services

- ✔ Parallel production lines ensure high uptime and operational flexibility

- ✔ Separation of security levels improves workflow efficiency

- ✔ Industrial-grade design suitable for continuous daily operation

- ✔ Optimized investment structure and long-term ROI for operators

This solution concept is intended for professional operators serving banks, central banks, and financial institutions.

### 🌐 Contact Us 联系我们

Shanghai YUEDU Technology Co., Ltd. | 上海越都科技有限公司

📧 Email: yuedushredders@gmail.com | denghao8410@gmail.com 
📱 WhatsApp / WeChat: +86 159 0178 2779
🌐 Website: www.yuedushredders.com

@YUEDUShredders – Empowering Global Industries with Intelligent Destruction Systems.

📞 Contact Us

Johnny Deng, Managing Director

☎️  Phone: +86 159 0178 2779 (WhatsApp / WeChat / Facebook / Telegram)

📧Email: denghao8410@gmail.com

Shanghai YUEDU Technology Co., Ltd. (YUEDU Shredders)

🌐 Website(Global): www.yuedushredders.com (note: VPN required for China access, as the server is hosted by Google.)

📧 Email: yuedushredders@gmail.com / denghao8410@gmail.com

☎️  Sales Supervisor: +86 159 0178 2779  (WhatsApp / WeChat / Facebook / Telegram) ☎️  Administrative Supervisor: +86 137 6138 5116

☎️  Head Office Main Line: +86 21 519 000 20

📍 Company Headquarters: Building 12, No. 555 Chengyin Road, Baoshan City Industrial Park, Shanghai, China