# 📄 Secure Document Destruction Solution – Normal & High-Security

## Integrated Shredding Lines – 2,000 Tons / Year

### 1. Project Overview

This project is designed for bank-related archival document destruction, combining operational efficiency and high security compliance.

- Total volume: approx. 2,000 tons / year

  - Normal documents: 1,500 tons / year

  - High-security documents (P6–P7): 500 tons / year

- Operation assumption:

  - 200 working days / year

  - 6 operating hours / day

- Site condition:

  - Single centralized location

  - Area ≥ 500 m², height approx. 6 m

- Design objective:

  - Stable, long-term industrial operation

  - Modular & parallel production lines

  - Clear separation of security levels

### 2. Overall System Concept

We recommend an integrated solution with separated security levels and parallel production lines:

- Normal documents and high-security documents are processed on independent lines

- Multiple parallel lines ensure:

  - Operational stability

  - Maintenance flexibility

  - No single-point failure risk

- All systems are centrally managed within one facility

## 3. Line A – Normal Document Shredding (EU P2–P3)

Annual volume: approx. 1,500 tons

- Required throughput: 1.2–1.3 tons/hour

- Configuration: 2 parallel shredding lines

Each line includes:

- Primary shredder

  - Motor power: 15 kW

  - Output size: approx. 20 mm

- Secondary shredder

  - Motor power: 10 kW

  - Security level: EU P2–P3

- Single-line capacity: 600–700 kg/hour

Discharge & baling:

- Shredded paper is sent directly to the customer’s existing baler

- Large paper bales for easy handling and resale

- No additional baler investment required

## 4. Line B – High-Security Document Shredding (EU P6–P7)

Annual volume: approx. 500 tons

- Required throughput: approx. 400–420 kg/hour

- Configuration: 2 parallel high-security shredding lines

Each line includes:

- Conveyor feeding system

- Primary shredder

  - Motor power: 11 kW

- High-security fine shredder

  - Motor power: 22 kW

  - Output size: 1–3 mm particles

  - Security level: EU P6–P7

- Single-line capacity: 150–200 kg/hour

Final treatment:

- Shredded paper powder is processed by a hydraulic briquetting system

- Compressed into small cylindrical briquettes

- Ensures maximum security and reduced volume

## 5. Key Advantages of the Integrated Solution

- ✔ Parallel line design for high reliability and redundancy

- ✔ Clear separation of normal and high-security workflows

- ✔ Industrial-grade operation, not an oversized single machine

- ✔ Suitable for long-term operation in banking and archival environments

🌐 Contact Us 联系我们

Shanghai YUEDU Technology Co., Ltd. | 上海越都科技有限公司

📧 Email: yuedushredders@gmail.com | denghao8410@gmail.com 
📱 WhatsApp / WeChat: +86 159 0178 2779
🌐 Website: www.yuedushredders.com

@YUEDUShredders – Empowering Global Industries with Intelligent Destruction Systems.

📞 Contact Us

Johnny Deng, Managing Director

☎️  Phone: +86 159 0178 2779 (WhatsApp / WeChat / Facebook / Telegram)

📧Email: denghao8410@gmail.com

Shanghai YUEDU Technology Co., Ltd. (YUEDU Shredders)

🌐 Website(Global): www.yuedushredders.com (note: VPN required for China access, as the server is hosted by Google.)

📧 Email: yuedushredders@gmail.com / denghao8410@gmail.com

☎️  Sales Supervisor: +86 159 0178 2779  (WhatsApp / WeChat / Facebook / Telegram) ☎️  Administrative Supervisor: +86 137 6138 5116

☎️  Head Office Main Line: +86 21 519 000 20

📍 Company Headquarters: Building 12, No. 555 Chengyin Road, Baoshan City Industrial Park, Shanghai, China